[Item: …………..]

Tripod turnstile pedestrian gate Magnetic FlowMotion® mTripod

Electric motor driven tripod turnstile for access control and passenger separation.

Function:

- Smoothly operating motor drive system for comfortable passage through gate

- Automatic motor driven turning after reception of release signal by gentle hand touch

- 2-way operation in entry and exit direction

- Suppression of release signal after set time if not used

- Pulse counting in both throughput directions

- Error history (logs) for technical service

- For indoor and outdoor applications

- Random generator for people and bag checks

Design:

- Design housing with an extremely slender silhouette, shapely form, flowing lines and frame and floor lighting (option)

- No visible connection elements or locks

- Decent colour design in basalt (sides) and anthracite (frame)

- Optional: customer-specific colours

Drive system/safety & security:

- Certified (TÜV Süd) low energy drive system (for maximum personal safety) with max. 1.69J, static opening and closing force max. 67N 
(according to prEN17352:2019, Performance Level d)ÖNORM EN 17352:2019-03-01

- Integrated MHTMTM drive system (brushless DC motor) for at least 10 million cycles (MCBF), certified according to EN 60335-2-103EN 60335-2-103:2015

- Duty cycle S1 (continuous operation), 100% according to DIN EN 60034-1OVE EN IEC 60034-1:2021-04-15

- Free passage in both directions in case of power outage or alarm signal

- Tripod gate transmits warning notification in case of forced entry

- Optional: under-climb and over-crawl detection

- Optional: drop-arm

- Optional: mechanical blocking

Electric/Software:

- Integrated MGC control system with display for parameterization and set-up

- Safety paths certified according to DIN EN 13849-1 Performance Level dÖNORM EN ISO 13849-1:2016-06-15

- Functions and parameters can be activated and parameterized as well as switched via I/O connections: passage direction, hold-open time, passage direction indication, illumination options, random generator, behaviour in case of alarm, notification in case of forced entry, over-climbing, under-crawling

- Connection for 100 – 240 VAC, 50/60 Hz

- Control with activation by reception (PULT02) as well as with all common card readers systems

- 8 digital inputs with free allocable functions

- 4 digital outputs and 6 output relays for illumination control and status readout (freely allocable)

- 5 connector slots for extension modules

- Connectable via Ethernet interface

Material:

- 2 component-polyurethane mDure

- Highly resistant to impacts, scratches and abrasion. UL tested according to UL0746C f1 (yellow card E 83364)

- Fire behaviour: tested according to UL-94 V0 and UL 0746C

- Resistant against corrosion, UV light, detergents and disinfectants

- Environmentally friendly disposal as household garbage

- Enclosure protection rating: IP54 (according to EN 60529)

Dimensions:

- Dimensions of housing:	1300 x 1050 x 290 mm (W x H x D) mTripod long

730 x 1050 x 290 mm – mTripod kurz

- Passage width: 		515 mm

- Installation space for readers:	60 x 80 x 40 mm (W x H x D)

Installation:

- Screw mounting on stable floor surface

- Alternative mounting on unfinished floor by use of adjustable foundation frame (see accessories)

- Alternative glue mounting by use of a metal base plate (see accessories)

- Paving stone assembly (see accessories)

- Platform stainless steel (see accessories)

Temperature range: 		-30 to +55 °C

Weight:				approx. 70 kg (mTripod long)

approx. 57 kg (mTripod short)

Installation execution by manufacturer trained installation specialist, including user training and hand-over of inspection manual, operating manuals, declaration of conformity according to EC machinery directive/ low voltage directive, EMC directive

Three-arm turnstile Magnetic FlowMotion® mTripod inkl. Verpackung

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	mTripod FMTP-ML – long version

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	mTripod FMTP-MS – short version

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	mTripod FMTPC-ML – long version with UL design

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	mTripod FMTPC-MS – short version with UL design

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option for three-arm turnstile Magnetic FlowMotion® mTripod

[Item: …………..]

OptionSpecial colour

Special colour for side panels, other elements in Anthracite

Manufacturer:	Magnetic Autocontrol GmbH

Typ: 		SL100

Colourtype: …………..    Number: …………..      Unit: pieces	Unit price: …………..	Total: …………..

[Item: …………..]

Option Sperrelement – Additional charge for carbon arms

For one mTripod (3 pcs.)

Manufacturer:	Magnetic Autocontrol GmbH

Typ: 		CS100MP

Colourtype: …………..    Number: …………..      Unit: pieces	Unit price: …………..	Total: …………..

Droparm / locking unit, only once per mTripod needed

[Item: …………..]

Option drop-arm mTripod, assembled, no EP required

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	DA100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option mechanical end-position locking +/- 5 degrees range of motion, assembled, no EP required

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	VERH100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Front plates/mounting plates

[Item: …………..]

Option Front Plate with RFID symbol (1 pc.)

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	FP104

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option Mounting frame for flush-mounted badge reader (d = 60 mm), bulk delivery (2 pcs., metal)

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	HUL100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Push-button

[Item: …………..]

Option illuminated push-button for release of passage by keystroke (per module and passage direction)

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	DT100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option mounting charge for accessories (once per option)

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	EP113

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Holder for external devices

For retrofit:

[Item: …………..]

Option supporting flange for the gate – for retrofit

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	LHAF100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Item: …………..]

Option fastening tube adjustable 115 -35 mm for reader device, plus EP113 – for retrofit

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	LHAR100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Factory assembly:

[Item: …………..]

Option fastening tube adjustable 115 -35 mm for reader device, plus EP113 – for factory assembly

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	LHAR100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option mounting charge for accessories (once per option)

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	EP113

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Accessory for reader device:

[Item: …………..]

Option standard top mounting for reader device - accessory for reader device

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	LHAP100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Coin validator

[Item: …………..]

Option Body housing empty

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	AUG200

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option Body housing, incl. coin validator mechanical

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	AUG201

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option Token, diameter 27 mm, with groove (50 pieces), for AUG201

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	CMZ012750

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Illumination

[Item: …………..]

Option Gate End Display – 2 pieces built-in per one passage

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	GED100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option Signal LED for indication of random generator trigger or GED extension red/green -> function has to be indicated at order, no power supply unit needed

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	DLED100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option extra charge for installation of illumination (once per option)

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	EP110

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option frame edge LED illumination red/green for long mTripod (ML version) -> plus EP110, power supply unit VE100 needed

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	RLED100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option frame edge LED illumination red/green for short mTripod (MS version) -> plus EP110, power supply unit VE100 needed

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	RLED101

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option floor LED illumination white for long mTripod version -> plus EP110, power supply unit VE100 needed

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	BOLED100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option floor LED illumination white for short mTripod version -> plus EP110, power supply unit VE100 needed

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	BOLED101

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option power supply unit, one VE100 can supply all electric options

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	VE100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option extra charge for installation of illumination (once per option)

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	EP110

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Sensors/buzzers

- power supply unit VE100

Option crawl-under detection for detection of an unauthorized passage below the locking arm 
(acoustic signal with ESU100 available), plus EP111

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	UKE100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option climb-over detection for detection of an unauthorized passageby climbing over the locking arm 
(acoustic signal with ESU100 available), plus EP111

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	USE100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option extra charge for installation of crawl-under/climb-over detection

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	EP111

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option internal buzzer, pulse signal/continuous signal, plus EP113

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	ESU100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option mounting charge for accessories (once per option)

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	EP113

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Interfaces

[Item: …………..]

Option Ethernet interface as plug-in module

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	EM01

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Service Module

[Item: …………..]

Option service module for updating the controller software and parameters

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	SM01

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

External desk panel

[Item: …………..]

Option desk panel 1 channel for the control of 1 device

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	PULT01

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option desk panel 1 channel for the control of 1 device

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	PULT01

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option desk panel 2 channel for the control of 2 devices

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	PULT02

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Foundation options

[Item: …………..]

Option alignment device for FlowMotion® (mTripod ML/MS, mWing)

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	ARS100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option 1: direct mounting

[Item: …………..]

Option mounting set for installation on finished floor surface

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	BSS100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option 2a: assembly on unfinished concrete floor ML

[Item: …………..]

Option foundation frame FlowMotion® long (ML), bridging structure for floor heating/floor screed and glue solution

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	FURA100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option threads and metal base plate

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	BSSFURA100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option 2b: assembly on unfinished concrete floor MS

[Item: …………..]

Option foundation frame FlowMotion® short (MS), bridging structure for floor heating/floor screed and glue solution

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	FURA101

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option threads and metal base plate

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	BSSFURA101

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option 3: glue based fixing

[Item: …………..]

Option foundation frame FlowMotion® long (ML), bridging structure for floor heating/floor screed and glue solution

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	FURA100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option foundation frame FlowMotion® short (MS), bridging structure for floor heating/floor screed and glue solution

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	FURA101

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option glue cartridge FlowMotion for fixing to a finished floor surface

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	BSSKL100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option 4: paving stone assembly

[Item: …………..]

Option foundation frame FlowMotion long (ML), for usage with paving stones, height = 150 mm

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	FURA102

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option foundation frame FlowMotion short (MS), for usage with paving stones, height = 150 mm

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	FURA104

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

[Item: …………..]

Option mounting set for installation on finished floor surface

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	BSS100

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..

Option 5: platform

[Item: …………..]

Option platform stainless steel, mTripod ML/MS, 1 passage

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	FMMS_TP

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..