High swing gates Magnetic with options and accessories

[Position: …………..]

Magnetic motorised swing gate, MPG362CA100

Motorised swing gate for barrier-free access, can be used in combination with a Magnetic turnstile MPT*.

Function:

- The MPG362* swing gate is designed for barrier-free access, primarily for users with wheelchairs, bicycles and goods deliveries.

- It is attached to the MPT* magnetic turnstile using the fastening materials supplied.

- After an activation signal (e.g. access control device), the swing gate opens 90° in the direction of passage.

- Opening and closing time approx. 3 seconds.

- 2-way operation in entry and exit direction

- The turnstile is supplied with a "voltage-free free-turning lock" as standard.

- The "locked when de-energised" configuration can be specified when ordering or easily converted in the field.

- Automatic closing after a set time

- Pulse counting in both directions of passage

- Error history (logs) for technical service

- In the event of forced passage, the barrier sends a warning message

- In the event of a power failure or alarm signal, the gate must be able to be opened freely in both directions

- Duty cycle S1 (continuous operation), 100% in accordance with DIN EN60034-1

- Passage cycles (MCBF) min. 10 million

- Safety and minimum distances in accordance with EN17352

Drive system/safety

- Certified (TÜV SÜD) low-energy drive (for maximum personal safety) up to max. 1.69J static force when opening and closing max. 67N

(in accordance with requirements for personnel barriers EN17352:2022, EN13849.1 		     Performance Level d cat.2)

- Integrated MHTMTM drive system (brushless DC motor with planetary gear) for at least 10 million passage cycles (MCBF), certified in accordance with EN 60335-2-103

Electrics/software:

- Integrated MGC control with display for parameterisation and commissioning

- Functions and parameters can be activated and adjusted and switched via digital inputs and outputs: direction of passage, hold-open time, direction of passage indicator, random generator

- Connection for 100–240 VAC, 50/60 Hz

- Control with activation via reception (PULT02) and with all common card readers

- 8 digital inputs with freely assignable functions

- 4 digital outputs and 6 output relays for status reporting (freely assignable)

- 5 plug-in card slots for expansion modules

- Can be networked via Ethernet interface

Design/material:

- Steel construction for attachment to Magnetic turnstile MPT* (support beam, service door, rotating wing, foot for rotating wing), hot-dip galvanised and then powder-coated in RAL7042 traffic grey

- Other RAL colours are available as an option

- High impact, scratch and abrasion resistance

- Rotating wing in stainless steel quality V4A (AISI316L; AISI316Ti) available as an option

- Guide brackets attached on both sides available as an option

- Guide brackets attached on both sides, including induction loops in the side panel, available as an option

- Rotary vane mounted on the floor with a low-maintenance PTFE plain bearing, 		        no relubrication required

Dimensions (without guide bars): 1451 x 500 x 2235 mm, (L x W x H)

Passage width: 	1087 mm

Temperature range: 		-30 to +55 °C

Weight:				approx. 220 kg (without options and accessories)

Protection class: 			IP 43 (IP54 available as an option)

Duty cycle (ED):			100

Designed for 10,000,000 person passages

Installation:

- Installation on load-bearing floor/foundation using composite anchors included in delivery

- Alternative installation using foundation frame (available as an accessory), required in conjunction with composite or paving stones

Installation by a fitter trained by the manufacturer

Instruction and handover of documentation including test book, operating instructions, declaration of conformity in accordance with the Machinery Directive / Low Voltage Directive / EMC Directive

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

Options for swing gate MPG362

[Position: …………..]

Special colour option

Complete steel construction of swing gate MPG362 powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option: Stainless steel swing gate

Stainless steel swing arm V4A (AISI316L; AISI316Ti), steel construction of the MPT362 swing gate powder-coated in RAL7042

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option: stainless steel swing gate, steel construction in special colour

Swing gate in stainless steel V4A (AISI316L; AISI316Ti), steel construction of the MPG362 swing gate powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option: Swing gate with guide rails on both sides for user guidance

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option: Swing gate with guide rails on both sides for user guidance, including induction loops in both side panels

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option profile half cylinder; PHZT3GBMP

Preparation of profile half cylinder on service door of support beam (MPT*3*/MPG/MPB)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

Accessories for swing gate MPG362

[Position: …………..]

Accessories for GED (Gate End Display) passage indicator; GED105MP

2 gate end displays (GED) for MPT*3*/MPG/MPB, green arrow, red X, one on each side, integrated into the support beam/hood

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: pcs		Packaging unit: …………..	GP: …………..

Lighting accessories; BE2LED

LED lighting on both sides for MPT*3* with MPG362/MPT*5*, with 4 lights, powder-coated in RAL7042, without twilight switch (not for mechanical high turnstiles, only for 230V/50Hz)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Lighting accessories; BE2LEDDS

LED lighting on both sides for MPT*3*/MPG362/MPT*5*, with 4 lights, powder-coated in RAL7042, with twilight switch (not for mechanical high turnstiles, only for 230V/50Hz)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; ADACHT3G16R

Raised roof for MPT*3* with MPG362, powder-coated in RAL 7042

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; ADACHT3G16FX

Raised roof for MPT*3* with MPG362, powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: ………   Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACHT3G16R

Free-standing pent roof for MPT*3* with MPG362, powder-coated in RAL7042 (6 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACHT3G16FX

Free-standing pent roof for MPT*3* with MPG362, powder-coated in special colour (6 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: ………   Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; BSS104

Mounting kit for pent roof on foundation frame, set for 2 posts

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; BSS105

Fastenings kit for pent roof on ground, set for 2 posts

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium foundation frame accessories; FURAT3G160

Foundation frame for MPT*3* with MPG362 without roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessories Premium foundation frame; FURAADT3G160

Foundation frame for MPT*3* with MPG362 with roof structure

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Premium foundation frame accessories; FURAT3G160D

Foundation frame for MPT*3* with MPG362 with pent roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Magnetic motorised swing gate, MPG372CA100

Motorised swing gate for barrier-free access and can be used as a stand-alone version.

Function:

- The MPG372* swing gate is designed for barrier-free access, primarily for users with wheelchairs, bicycles and goods deliveries.

- Intended for use as a free-standing, stand-alone model.

- After an authorisation signal (e.g. access control device), the rotating wing opens 90° in the direction of passage.

- Opening and closing time approx. 3 seconds

- 2-way operation in entry and exit direction

- The swing gate is supplied with "voltage-free free-swinging lock" as standard.

- The "locked when de-energised" configuration can be specified when ordering or easily converted in the field.

- Automatic closing after a set time

- Pulse counting in both directions of passage

- Error history (logs) for technical service

- In the event of forced passage, the barrier sends a warning message

- In the event of a power failure or alarm signal, the gate can be opened freely in both directions

- Duty cycle S1 (continuous operation), 100% in accordance with DIN EN60034-1

- Passage cycles (MCBF) min. 10 million

- Safety and minimum distances in accordance with EN17352

Drive system/safety

- Certified (TÜV SÜD) low-energy drive (for maximum personal safety) up to max. 1.69J static force when opening and closing max. 67N

(in accordance with requirements for personnel barriers EN17352:2022, EN13849.1 		     Performance Level d cat.2)

- Integrated MHTMTM drive system (brushless DC motor with planetary gear) for at least 10 million passage cycles (MCBF), certified in accordance with EN 60335-2-103

Electrics/software:

- Integrated MGC control with display for parameterisation and commissioning

- Functions and parameters can be activated and adjusted and switched via digital inputs and outputs: direction of passage, hold-open time, direction of passage indicator, random generator

- Connection for 100–240 VAC, 50/60 Hz

- Control with activation via reception (PULT02) and with all common card readers

- 8 digital inputs with freely assignable functions

- 4 digital outputs and 6 output relays for status reporting (freely assignable)

- 5 plug-in card slots for expansion modules

- Can be networked via Ethernet interface

Design/material:

- Steel construction as a free-standing single unit (support beam, service door, hinged door, foot for hinged door, foot for support beam), hot-dip galvanised and then powder-coated in RAL7042 traffic grey

- Other RAL colours are available as an option

- High impact, scratch and abrasion resistance

- Swing arm in V4A stainless steel quality (AISI316L; AISI316Ti) available as an option

- Guide brackets attached on both sides available as an option

- Guide brackets attached on both sides, including induction loops in the side panel, available as an option

- Rotary vane mounted on the floor with a low-maintenance PTFE plain bearing, 		        no relubrication required

Dimensions (without guide brackets): 1623 x 500 x 2235 mm, (L x W x H)

Passage width:                        1070 mm

Temperature range: 		-30 to +55 °C

Weight:				approx. 250 kg (without options and accessories)

Protection class: 			IP 43 (IP54 available as an option)

Duty cycle (ED):		100

Designed for 10,000,000 person passages

Installation:

- Installation on load-bearing floor/foundation using composite anchors included in delivery

- Alternative installation using foundation frame (available as an accessory), required in conjunction with composite or paving stones

Installation by a fitter trained by the manufacturer

Instruction and handover of documentation including test book, operating instructions, declaration of conformity in accordance with the Machinery Directive / Low Voltage Directive / EMC Directive

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

Options for swing gate MPG372

[Position: …………..]

Special colour option

Complete steel construction of swing gate MPG372 powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option: Stainless steel swing gate

Stainless steel swing gate V4A (AISI316L; AISI316Ti), steel construction of the MPG swing gate 372 powder-coated in RAL7042

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option: stainless steel swing gate, steel construction in special colour

Swing gate in stainless steel V4A (AISI316L; AISI316Ti), steel construction of the MPG372 swing gate powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option: Swing gate with guide rails on both sides for user guidance

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option: Swing gate with guide rails on both sides for user guidance, including induction loops in both side panels

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option profile half cylinder; PHZT3GBMP

Preparation of profile half cylinder on service door of support beam (MPT*3*/MPG/MPB)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

Accessories for swing gate MPG372

[Position: …………..]

Accessories for GED (Gate End Display) passage indicator; GED105MP

2 gate end displays (GED) for MPT*3*/MPG/MPB, green arrow, red X, one on each side, integrated into the support beam/hood

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: pcs		Packaging unit: …………..	GP: …………..

Lighting accessories; BELED

LED lighting on both sides for MPT*3*/MPG/MPB, with 2 lights, powder-coated in RAL7042, without twilight switch (not for mechanical high turnstiles, only for 230V/50Hz)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Lighting accessories; BELEDDS

LED lighting on both sides for MPT*3*/MPG/MPB*, with 2 lights, powder-coated in RAL7042, with twilight switch (not for mechanical high turnstiles, only for 230V/50Hz)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessory; ADACHG17R

Raised roof for MPG372, powder-coated in RAL 7042

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessory; ADACHG17FX

Raised roof for MPG372, powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: ………   Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessory; PDACHG17R

Free-standing pent roof for MPG372, powder-coated in RAL7042 (4 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessory; PDACHG17FX

Free-standing pent roof for MPG372, powder-coated in special colour (4 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: ………   Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; BSS104

Mounting kit for pent roof on foundation frame, set for 2 posts

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; BSS105

Fastenings kit for pent roof on ground, set for 2 posts

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessories Premium foundation frame; FURAT3G160

Foundation frame for MPT*3* with MPG362 without roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessories Premium foundation frame; FURAG170

Foundation frame for MPG372 without roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessories Premium foundation frame; FURAG170D

Foundation frame for MPG372 with pent roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..