Turnstiles (electromechanical) with bicycle gate (motorised) Magnetic with options and accessories

[Position: …………..]

Electromechanical turnstile with motorised bicycle door Magnetic, MPB411*

Electromechanical turnstile for access control and single passage in combination with a motorised bicycle door for transporting a bicycle.

Function:

- The turnstile is used to control pedestrians, mainly in outdoor areas with relatively high security requirements.

- Smooth-running electromechanics for comfortable passage

- Spring-assisted rotary movement after release impulse and light touch of the hand

- 2-way operation in entry and exit directions.

- The turnstile is supplied with a "voltage-free free-rotating lock" as standard.

- The "locked when de-energised" configuration can be specified when ordering or easily converted in the field.

- Signal deletion when not in use after a set time

- Pulse counting in both directions of passage

- Error history (logs) for technical service

- Integrated random generator for person and bag checks

- In the event of forced passage, the barrier sends a warning message

- In the event of a power failure or alarm signal, the barrier can be rotated freely in both directions

- Duty cycle S1 (continuous operation), 100% in accordance with DIN EN60034-1

- Passage cycles (MCBF) min. 10 million

- Safety and minimum distances in accordance with EN17352

- The bicycle door is used to carry a bicycle when using the turnstile

- The turnstile side and bicycle door side are separated by a guide element made of V4A stainless steel (AISI316L; AISI316Ti)

- The presence of a bicycle can be detected by induction loops (installed in the floor) provided by the customer, or the bicycle door can be released by an additional button

Electrics/software:

- Integrated MGC control with display for parameterisation and commissioning

- Functions and parameters can be activated and adjusted and switched via digital inputs and outputs: direction of passage, hold-open time, direction of passage indicator, random generator

- Connection for 100–240 VAC, 50/60 Hz

- Control with activation by reception (PULT02) and with all common card readers

- 8 digital inputs with freely assignable functions

- 4 digital outputs and 6 output relays for status reporting (freely assignable)

- 5 plug-in card slots for expansion modules

- Can be networked via Ethernet interface

Drive system/safety (bicycle door)

- Certified (TÜV SÜD) low-energy drive (for maximum personal safety) up to max. 1.69J static force when opening and closing max 67N

(in accordance with requirements for personnel barriers EN17352:2022, EN13849.1 		     Performance Level d cat.2)

- Integrated MHTMTM drive system (brushless DC motor with planetary gear) for at least 10 million passage cycles (MCBF), certified in accordance with EN 60335-2-103

Design/material:

- Self-supporting steel construction (support beams, service door, centre column, cage half, locking comb, bicycle door), hot-dip galvanised and then powder-coated in RAL7042 traffic grey

- Guide element in V4A stainless steel quality (AISI316L; AISI316Ti)

- Other RAL colours are available as an option

- High impact, scratch and abrasion resistance

- Centre column and bicycle door in stainless steel quality V4A (AISI316L; AISI316Ti) available as an option

- Floor mounting of the centre column and bicycle door with a low-maintenance PTFE plain bearing, no relubrication required

Dimensions:                             2500 x 2458 x 2235 mm, (L x W x H)

Temperature range: 		-30 to +55 °C

Weight:				approx. 550 kg (without options and accessories)

Protection class: 			IP 43 (IP54 available as an option)

Duty cycle (ED):			100

Designed for 10,000,000 passenger passages

Throughput

(Depending on reader, product configuration and user behaviour): approx. 10 people per minute1

Installation

- Installation on load-bearing floor/foundation, composite anchors (available as accessories)

- Alternative installation using foundation frame (available as an accessory), required in conjunction with composite or paving stones

Installation by a fitter trained by the manufacturer,

Instruction and handover of documentation including test book, operating instructions, declaration of conformity according to Machinery Directive / Low Voltage Directive / EMC Directive

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

Options for turnstile with bicycle gate MPB411*

[Position: …………..]

Left-hand version option MPB411A*

Viewed from the public area, the bicycle door is on the left-hand side (turnstile with passage on the left)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Right-hand version option MPB411B*

Viewed from the public area, the bicycle door is on the right-hand side (turnstile with passage on the right)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Special colour option

All parts powder-coated in special colour, guide element in stainless steel (V4A/AISI316L; AISI316Ti), prepared for on-site induction loops

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: pcs		EP: …………..	GP: …………..

[Position: …………..]

Stainless steel option

All parts powder-coated in RAL7042, centre column, bicycle door and guide element in stainless steel (V4A/AISI316L; AISI316Ti), prepared for on-site induction loops

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

Special colour/stainless steel option

All parts powder-coated in special colour, centre column, bicycle door and guide element in stainless steel (V4A/AISI316L; AISI316Ti), prepared for on-site induction loops

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option profile half cylinder; PHZT3GBMP

Preparation of profile half cylinder on service door of support beam (MPT*3*/MPG/MPB)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Locking option (MPT333*); EP100

Conversion fee Input locked in the event of a power failure

Required once per direction of travel

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Locking option (MPT333*); EP101

Conversion fee Output locked in the event of a power failure

Required once per direction of travel

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Locking option (MPT333*); MAENT101

MPT mechanical unlocking lock (per direction)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

Accessories for MPB411 turnstile*

[Position: …………..]

Accessories for GED (Gate End Display) passage indicator; GED105MP

2 gate end displays (GED) for MPT*3*/MPG/MPB, green arrow, red X, one on each side, integrated into the support beam/cover

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: pcs		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessory mounting column for the installation of access controls; ASMP

Mounting column made of chrome steel, powder-coated RAL7042, incl. mounting plate

Dimensions: 704 x 260 x 260 mm, installation area max. 600 x 160 mm

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessory mounting column for the installation of access controls; ASMPX

Mounting column made of chrome steel, powder-coated in RAL special colour, incl. mounting plate

Dimensions: 704 x 260 x 260 mm, installation area max. 600 x 160 mm

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessory mounting column for the installation of access controls; ASMPPHZ

Chrome steel mounting column, powder-coated RAL7042, incl. mounting plate, prepared for profile half cylinder (PHZ)

Dimensions: 704 x 260 x 260 mm, installation area max. 600 x 160 mm

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessory mounting column for the installation of access controls; ASMPPHZX

Chrome steel mounting column, powder-coated in RAL special colour, incl. mounting plate, prepared for PHZ, dimensions: 704 x 260 x 260 mm, installation area max. 600 x 160 mm

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessories Mounting bracket kit; AMWMPB

Mounting bracket mounting kit for cage half MPB in uncoated stainless steel, 1 piece

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: pcs		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessories Mounting bracket kit; AMWMPBR

Mounting bracket kit for cage half MPB in stainless steel, coated in RAL 7042, 1 piece

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: pcs		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessories Mounting bracket kit; AMWMPBX

Mounting bracket kit for cage half MPB in stainless steel, same colour as turnstile, 1 piece

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: pcs		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessories Push button; DT101

Illuminated push button for enabling passage by pressing the button

(per passage direction)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Item: …………..]

Lighting accessories; BELED

LED lighting on both sides for MPT*3*/MPG/MPB, with 2 lights, powder-coated in RAL7042, without twilight switch (not for mechanical high turnstiles, only for 230V/50Hz)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Lighting accessories; BELEDDS

LED lighting on both sides for MPT*3*/MPG/MPB, with 2 lights, powder-coated in RAL7042, with twilight switch (not for mechanical high turnstiles, only for 230V/50Hz)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; ADACHBAR

Raised roof for MPB (left-hand version), powder-coated in RAL 7042

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; ADACHBAFX

Raised roof for MPB (left-hand version), powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; ADACHBBR

Raised roof for MPB (right-hand version), powder-coated in RAL 7042

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; ADACHBBFX

Raised roof for MPB (right-hand version), powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACHBR

Free-standing pent roof for MPB, powder-coated in RAL7042 (4 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACHBFX

Free-standing pent roof for MPB, powder-coated in special colour (4 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACH2BR

Detached pent roof for 2 MPB, powder-coated in RAL7042 (6 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACH2BFX

Free-standing pent roof for 2 MPB, powder-coated in special colour (6 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; BSS104

Mounting kit for pent roof on foundation frame, set for 2 posts

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; BSS105

Fastenings kit for pent roof on ground, set for 2 posts

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessories Premium foundation frame; FURAB

Foundation frame set for MPB without roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Premium foundation frame accessories; FURABD

Foundation frame set for MPB with pent roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessories Premium foundation frame; FURA2B

Foundation frame set for 2x MPB without roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Premium foundation frame accessories; FURA2BD

Foundation frame set for 2x MPB with pent roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessories Premium foundation frame; MPBBS

Fastening kit on foundation frame (1xMPB)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

Accessories Mounting kit; MPBBSOF

Mounting kit for installation on concrete foundation (1xMPB)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..