[Item: …………..]

Access barrier Magnetic MHTM™ MicroDrive, type Access XXL

Electromotive-operated traffic barrier for vehicle access control, entry/exit for passenger cars / trucks, blocking width up to 10.0 m, opening and closing times 8.0 s or more.

Straight barrier boom, suitable for barrier boom skirts with / without climb-over protection.

Drive:

Integrated MHTM™ MicroDrive drive system for at least 3 million passage cycles (MCBF)

Highly dynamic and energy-efficient BLDC drive 24 V

Additional lever mechanism for locking the barrier boom in both end positions

Low maintenance and virtually noiseless

Lowest dynamic and static impact forces

Highest positioning accuracy without bouncing in the end positions

Design:

Elegant aluminium housing, chromated and powder-coated

Inner carrier frame zinc-phosphated and cathodically dip-coated

Stainless steel base frame

Standard colour version side panels and hood in Yellow orange (RAL 2000), doors in Black grey (RAL 7021)

Optionally available in RAL colour combinations according to the client's wishes

Safety:

Safe control according to EN 13849, Performance Level C

Testable input for safety light barrier or laser scanner as personal protection,
functional test before each closing movement

Integrated 2-channel loop detector, pluggable module version, multiplex operation, frequency range (high / low) can be switched via menu, display of current frequency and frequency change when driven over

Service switch for technicians for secure blocking of the barrier boom in both end positions, movement of the barrier boom still possible via 2 keys on the control unit

Electronic impact detection with adjustable reversing behaviour

Lever mechanism largely protected from crushing and shearing points

Soft edge protection under the barrier boom

Control:

Control unit MGC Pro with extended functional scope

Large graphic and clear text display

4 Control buttons

Configuration via self-explanatory menu control

8 digital inputs 24 V / 10 mA

4 digital outputs 24 V / 100 mA

6 relay outputs 30 V / 1 A

4 free module slots for optional installation of: second 2-channel loop detector module for extension to 4 loops, radio module, Ethernet module, RS485 module, CAN module (with vehicle counter)

Adjustable functions:

8 operating modes (4 manual, 4 automatic)

Opening and closing times in 3 steps

Free configuration of the digital inputs and outputs and the relay outputs with numerous available functions

Loop detector functionality

Reaction to power failure: locked, unlocked

Reverse after impact and subsequent reaction

Start-up behaviour (8 options)

Closing delay

Hold-open time up to 60 s

Various cut off angles

Opening vend count

System extension master / slave operation

Traffic light control

Scope of delivery:

Prohibition signs for pedestrian passage and prohibition for motorised and non-motorised bicycles, documentation, U-profiles for attachment.

fulfilled standards:

EN ISO 12100, EN 60204, EN 61000-6-2, EN 61000-6-3, EN ISO 13849, EN 13241, EN 12453, 
testing in accordance with the Construction Products Regulation

Technical data:

| Power supply: | 85 – 264 AC, 50/60 Hz |
| --- | --- |
| Power consumption max: | 30 W |
| Duty cycle: | 100 % (operating mode: S1) |
| Specified number of cycles: | 10 Mio. million openings / closings |
| Opening times: | 8,0 s / 10,0 s / 12,0 s |
| Closing times: | 8,0 s / 10,0 s / 12,0 s |
| Temperature range: | –30° C to +55° C |
| Dimensions (W x D x H): | 435 mm x 360 mm x 1.164 mm |
| IP rating: | IP 54 |
| Weight: | 112 kg |

Assembly:

On on-site concrete foundation using U-profiles (included in delivery) and 4 adhesive anchors (can be ordered separately)

A pendulum support or a support post is always required

Installation by a technician certified by the manufacturer

Instruction and handover of documentation including test book, Declaration of Conformity according to Machinery Directive / Low Voltage Directive / EMC Directive and Declaration of Performance according to Construction Products Regulation

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | Access XXL, Yellow orange (RAL 2000) |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Barrier boom MicroBoom-XL with blocking width ........... m (max. 10.0 m)

Barrier booms up to 6 m blocking width: Octagonal aluminium profile 121 x 86 mm with soft edge protection, red reflective stripes on both sides every 500 mm.

Barrier booms with blocking width greater than 6 m: Rear boom section 5 m long with octagonal aluminium profile 121 x 86 mm, then tapered to front boom section with octagonal aluminium profile 100 x 55 mm, generally with soft edge protection, red reflection stripes on both sides every 500 mm.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MicroBoom-XLxxx |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

Options for access barrier Magnetic MHTM™ MicroDrive, type Access XXL

[Item: …………..]

Option special colour SL01

Barrier housing and hood in the same freely selectable colour: RAL ………….. (please fill in).

Doors in standard colour (Black grey, similar to RAL 7021).

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | SL01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option special colour SL02

Barrier housing, hood and doors in one freely selectable colour: RAL ………….. (please fill in).

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | SL02 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option special colour SL03

Barrier housing and hood in a freely selectable colour: RAL ………….. (please fill in).

Doors in other freely selectable colour: RAL ………….. (please fill in).

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | SL03 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option special colour SL04

Barrier housing in a freely selectable colour: RAL ………….. (please fill in).

Hood in freely selectable colour: RAL ………….. (please fill in).

Doors in freely selectable colour: RAL ………….. (please fill in).

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | SL04 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option attachment set BSS50

Installation material for screwing a barrier to the concrete foundation, consisting of 4 adhesive cartridges and screw-connection M12 with internal thread.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | BSS50 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option attachment set BSS51

Installation material for screwing a barrier to the concrete foundation, consisting of 4 adhesive cartridges and screw-connection M12 with external thread.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | BSS51 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Standard boom skirt for installing under the barrier boom GSxx000_2000

Installation of the boom skirt under the barrier boom as under-crawl protection. Folds up when the barrier is opened. Patented skirt is free of shearing and crushing points. The boom skirt bars are individually exchangeable. A soft edge protector is installed below the skirt and is easily replaceable.

Maximum blocking width is 10.0 m.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | GSxx000_2000 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Boom skirt with 1.3 m high climb-over protection for installing laterally on the barrier boom
GUE0x000_2000

Installation of the boom skirt to the side of the barrier boom as climb-over and crawl-under protection. Folds up when the barrier is opened. Patented skirt is free of shearing and crushing points. The boom skirt bars are individually exchangeable. A soft edge protector is installed below the skirt and is easily replaceable.

Maximum blocking width is 6.0 m.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | GUE0x000_2000 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Boom skirt with 1.8 m high climb-over protection for installing laterally on the barrier boom
GUE0x000_2001

Installation of the boom skirt to the side of the barrier boom as climb-over and crawl-under protection. Folds up when the barrier is opened. Patented skirt is free of shearing and crushing points. The boom skirt bars are individually exchangeable. A soft edge protector is installed below the skirt and is easily replaceable.

Maximum blocking width is 6.0 m.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | GUE0x000_2001 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option second loop detector module 2-channel DM02

Pluggable module for expansion of the MGC (Pro) control to 4 loop channels,
multiplex of all 4 loops without bilateral cross talk

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | DM02 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option Ethernet module EM01

Pluggable Ethernet module for communication via MODBUS TCP/IP protocol.

Control and parameterisation of MHTM™ MicroDrive barriers, as well as export of parameters, information and operating states possible.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | EM01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option RS485 module RSM01

Pluggable RS485 module for communication via MODBUS protocol.

Control and parameterisation of MHTM™ MicroDrive barriers, as well as export of parameters, information and operating states possible.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | RSM01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option CAN module ECN01

Pluggable CAN module for communication via CANOpen protocol.

Used for counting vehicles in master-slave operation of several barriers.

Control and parameterisation of MHTM™ MicroDrive barriers, as well as export of parameters, information and operating states possible.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | ECN01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option radio module FM02

Pluggable receiver module, FM-modulated, 433 MHz.

Management of up to 500 hand transmitters.

Hand transmitter individually erasable in case of loss or theft of a transmitter.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | FM02 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option remote control 1-channel HS1K01

Remote control 1-channel, FM-modulated, 433 MHz.

To open / close a barrier in operating mode 3.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | HS1K01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option remote control 2-channel HS2K01

Remote control 2-channel, FM-modulated, 433 MHz.

For opening / closing up to two barriers in operating mode 3 or one barrier in operating mode 4.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | HS2K01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option remote control 4-channel HS4K01

Remote control 4-channel, FM-modulated, 433 MHz.

For opening / closing up to four barriers in operating mode 3 or up to two barriers in operating mode 4.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | HS4K01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option mini hand transmitter 2-channel MHS2K01

Remote control 2-channel, FM-modulated, 433 MHz.

For opening / closing a barrier in operating mode 4 or for opening / closing two barriers in operating mode 3.

With ring to carry on your key ring.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MHS2K01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option LED illumination strip red / green for barrier boom LEDS60/61

Including supply unit, control components and mounting plate

The LED illumination strips (red / green) are installed along the entire length of the underside of the barrier boom. Control is carried out by the barrier control unit:

Continuous green when barrier is open, flashing red during opening or closing, continuous red when barrier is closed

Not possible in combination with standard boom skirts GS0x000_2000!

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | LEDS60/61 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option red LED lights for barrier boom LEDX04

Including supply unit, control components and assembly plate.

Two red LED lights are installed on each side of the barrier boom.

Function adjustable on the control unit.

Not possible in combination with boom skirts with climb-over protection GUE0x000_2000 or GUE0x000_2001!

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | LEDX04 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option red LED lights for barrier boom LEDX06

Including supply unit, control components and assembly plate.

Three red LED lights are installed on each side of the barrier boom.

Function adjustable on the control unit.

Not possible in combination with boom skirts with climb-over protection GUE0x000_2000 or GUE0x000_2001!

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | LEDX06 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option red LED lights for barrier boom LEDX10

Including supply unit, control components and assembly plate.

Five red LED lights are installed on each side of the barrier boom.

Function adjustable on the control unit.

Not possible in combination with boom skirts with climb-over protection GUE0x000_2000 or GUE0x000_2001!

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | LEDX10 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option pendulum support for barrier boom PS01

Aluminium, powder-coated RAL 9010 (Pure white), infinitely height-adjustable (-50 mm / +145 mm).

Installed on barrier boom.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | PS01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option support post for barrier boom AP51

For blocking widths up to 6.0 m

Galvanised, powder-coated RAL 7043 (Traffic grey) with infinitely height-adjustable
(–40 mm / +160 mm) boom tip support and boom guide.

To be installed on on-site foundation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | AP51 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option support post for barrier boom AP01

For blocking widths > 6.0 m

Galvanised, powder-coated RAL 7043 (Traffic grey) with infinitely height-adjustable
(-40 mm / +160 mm) boom tip support and boom guide.

To be installed on on-site foundation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | AP01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option electro-mechanical barrier boom locking BV50/51

Locking of the barrier boom when the barrier is closed via electromagnetically operated bolts, installed in the barrier boom. Supply unit and control components included.

In addition, a support post for barrier boom with locking unit APV01 or APV51 is required.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | BV50/51 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option support post for barrier boom with locking unit APV51

For blocking widths up to 6.0 m

Galvanised, powder-coated RAL 7043 (Traffic grey) with infinitely height-adjustable boom tip support (-40 mm / +160 mm) and boom guide. Prepared for electro-mechanical locking.

Installed on on-site foundation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | APV51 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option support post for barrier boom with locking unit APV01

For blocking widths > 6.0 m

Galvanised, powder-coated RAL 7043 (Traffic grey) with infinitely height-adjustable boom tip support (-40 mm / +160 mm) and boom guide. Prepared for electro-mechanical locking.

Installed on on-site foundation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | APV01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option attachment set BSAP01

Installation material for screwing a support post to the concrete foundation, consisting of adhesive cartridges and screwing parts.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | BSAP01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option key switch SS51

For permanent open / button function CLOSE.

Installed in the barrier housing. Design for on-site profile cylinder.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | SS51 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option GERMAN fire brigade switch FWS51

For "permanent open" with actuating element.

German version, according to DIN 3223 (triangular hydrant key) for opening the barrier.

Usable by fire brigade, ambulance, emergency doctor, etc. in case of fire or danger

Installed in the barrier housing.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | FWS51 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Optional safety light barrier for pedestrian protection LS21

With test input for connection to control unit MGC (Pro).

Tested before each closing movement to verify proper operation.

Consisting of transmitter and receiver incl. attachments.

Installed on barrier housing and support posts (must be ordered separately).

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | LS21 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………….]

Option radar sensor as replacement for safety and opening loop RADS01

With test input for connection to control unit MGC (Pro)

Tested before each closing movement to verify proper operation.

Safe protection for pedestrians under the closing barrier boom due to variably adjustable size of the safety zone.

Reliable detection of vehicles (cars, lorries, trucks).

Replacement of induction loops by adjustable field size of safety and separate opening zone. Time saving compared to laying induction loops, as no floor laying is necessary.

Reliable detection of moving and stationary objects and persons. Automatic teach in of the environment.

Adjustable blanking of pedestrians in the opening zone.

Non-contact protective device for personal protection in accordance with EN 12453 (device with minimum protection level D). Only possible in conjunction with MicroDrive barriers with a closing time greater than 2.2 s (dynamic impact forces < 400 N = minimum protection level C).

Horizontal field size up to 6.0 m x 6.0 m. Installation in attachment housing for installing to a barrier housing or alternative installation in separate installation post.

When used with ACCESS XXL, it is only possible up to a blocking width of 6.0 m.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | RADS01 |  |  |  |
| Quantity: | ……………… | Piece(s) | UP: …………… | TP: …………… |

[Item: …………….]

Option mounting bracket radar sensor RADG01

For mounting the radar sensor RADS01 on the barrier housing.

Cable routing protected inside the attachment housing

Aluminium design with powder coating RAL 7043.

Can be mounted on the left or right side of the barrier housing.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | RADG01 |  |  |  |
| Quantity: | ……………… | Piece(s) | UP: …………… | TP: …………… |

[Item: …………….]

Option mounting wedge 30° for radar sensor RADMK01

Recommended for use when radar sensor is also used for opening or presence field.

Die-cast aluminium design.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | RADMK01 |  |  |  |
| Quantity: | ……………… | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option horizontal laser scanner for pedestrian protection MLSHOR01

or as replacement for safety and opening loops.

With test input for connection to control unit MGC (Pro).

Tested before each closing movement to verify proper operation.

Safe protection for pedestrians under the closing barrier boom due to variably adjustable size of the safety zone.

Reliable detection of vehicles by using reference point (e.g. wall, building, separate post).

Replacement of induction loops by adjustable field size of safety and separate opening zone. Time saving compared to laying induction loops, as no floor laying is necessary.

Reliable detection of moving and stationary objects and persons. Automatic teach in of the environment.

Adjustable blanking of pedestrians in the opening zone.

TÜV-approved in accordance with EN 12453 as non-contact protective device for pedestrian protection (device with minimum protection level E).

Horizontal field size up to 9.9 m x 9.9 m. Installation in attachment housing for installing to a barrier housing or alternative installation in separate installation post.

When used with boom skirts, one laser scanner each must be installed to the right and left of the barrier boom with field direction away from the barrier boom, since the laser scanner recognises the boom skirt as an obstacle.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MLSHOR01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option attachment housing laser scanner MLGHOR01

For attaching the MLSHOR01 laser scanner on the barrier housing.

Cover of the laser scanner to protect against vandalism, cable routing secured inside the attachment housing.

Stainless steel version with powder coating RAL 7043.

Not when using boom skirts!

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MLGHOR01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option installation post for laser scanner MLPEIN01

For separate installation of the laser scanner MLSHOR01 next to the barrier.

Laser scanner cover to protect against vandalism, extremely resilient.

Stainless steel version with powder coating RAL 7043.

Installation on on-site concrete foundation by means of 2 adhesive anchors.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MLPEIN01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option reference post for laser scanner MLPREF01

For use as a reference point for the MLSHOR01 laser scanner when no wall or other object can be used as a reference point.

Stainless steel version with powder coating RAL 7043.

Installation on on-site concrete foundation by means of 2 adhesive anchors.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MLPREF01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option attachment set BSS01

Installation material for screwing a terminal / a post to the concrete foundation, consisting of 4 adhesive cartridges and screwing parts.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | BSS01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option vertical laser scanner for vehicle protection MLSVER01

Safe protection even for special vehicles that cannot be reliably detected by induction loops. Example: Truck or agricultural vehicle with excess overhang or detection of drawbars between towing vehicle and trailer. Prevents the barrier boom from closing for as long as an object is in the vertical detection field.

Adjustable width of the safety zone. Teach-in of the ground as reference, no separate reference point necessary.

With test input for connection to control unit MGC (Pro).

Tested before each closing movement to verify proper operation.

Reliable detection of moving and stationary objects and persons. Can therefore also be used instead of a safety light barrier. When used with barrier grids, one laser scanner each must be mounted to the right and left of the barrier boom.

TÜV-approved in accordance with EN 12453 as non-contact protective device for pedestrian protection.

Vertical field size up to 9.9 m x 9.9 m. Can be installed in recessed housing for wall attachment or for installing on separate post.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MLSVER01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option installation housing laser scanner MLGVER_0000

For attaching the laser scanner MLSVER01 on a wall or on a separate post MLPF01. The laser scanner can be installed on the left or right side of the housing, so it can be used on the left or right side of the barrier boom.

Laser scanner cover to protect against vandalism.

Stainless steel version with powder coating RAL 7043.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MLGVER_0000 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option post for laser scanner MLPF01

For separate installation of the laser scanner MLSVER01 by means of the installation housing MLGVER_0000 next to the barrier.

Installation on on-site concrete foundation by means of 2 adhesive anchors.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MLPF01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option attachment set BSS01

Installation material for screwing a terminal / a post to the concrete foundation, consisting of 4 adhesive cartridges and screwing parts.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | BSS01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option induction loop IS12
as safety loop for cars

Circumference 12 m incl. waterproof connection box and 15 m supply line, laying loops and connection to previously described barrier controller, incl. measurements and documentation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | IS12 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option induction loop IS15
as safety loop for trucks

Circumference 15 m incl. waterproof connection box and 15 m supply line, laying loops and connection to previously described barrier controller, incl. measurements and documentation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | IS15 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option induction loop IS21
as safety loop for trucks

Circumference 21 m incl. waterproof connection box and 15 m supply line, laying loops and connection to previously described barrier controller, incl. measurements and documentation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | IS21 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option induction loop IS12
as opening loop for cars

Circumference 12 m incl. waterproof connection box and 15 m supply line, laying loops and connection to previously described barrier controller, incl. measurements and documentation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | IS12 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option induction loop IS15
as opening loop for trucks

Circumference 15 m incl. waterproof connection box and 15 m supply line, laying loops and connection to previously described barrier controller, incl. measurements and documentation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | IS15 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option induction loop IS21
as opening loop for trucks

Circumference 21 m incl. waterproof connection box and 15 m supply line, laying loops and connection to previously described barrier controller, incl. measurements and documentation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | IS21 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |