[Item: …………..]

Toll barrier Magnetic MHTM™ MicroDrive, type Toll HighSpeed 2

Electromotive-operated traffic barrier for handling vehicles, on motorways, blocking width up to 1.5 m, opening and closing times 0.3 s or more.

Drive:

Integrated MHTM™ MicroDrive drive system for at least 10 million passage cycles (MCBF)

Highly dynamic and energy-efficient BLDC drive 48 V

Additional lever mechanism for locking the barrier boom in both end positions

Low maintenance

Lowest dynamic and static impact forces

Highest positioning accuracy without bouncing in the end positions

Design:

Elegant and internationally awarded aluminium housing, chromated and powder-coated

Stainless steel base frame

Standard colour version side panels and hood in Yellow orange (RAL 2000), doors in Black grey (RAL 7021)

Optionally available in RAL colour combinations according to the client's wishes

Safety:

Safe control according to EN 13849, Performance Level C

Testable input for safety light barrier or laser scanner as personal protection,
functional test before each closing movement

Integrated 2-channel loop detector, pluggable module version, multiplex operation, frequency range (high / low) can be switched via menu, display of current frequency and frequency change when driven over

Service switch for technicians for secure blocking of the barrier boom in both end positions, movement of the barrier boom still possible via 2 keys on the control unit

Electronic impact detection with adjustable reversing behaviour

Lever mechanism largely protected from crushing and shearing points

Control:

Large graphic and clear text display

4 Control buttons

Configuration via self-explanatory menu control

8 digital inputs 24 V / 10 mA

4 digital outputs 24 V / 100 mA

6 relay outputs 30 V / 1 A

4 free module slots for optional installation of: second 2-channel loop detector module for extension to 4 loops, radio module, Ethernet module, RS485 module, CAN module (with vehicle counter)

Note for planners: There are 2 different alternatives available for the control unit. The following description highlights the differences. Alternative 1: Control unit MGC without loop detector module Alternative 2: Control unit MGC with 2-channel loop detector module

Adjustable functions:

8 operating modes (4 manual, 4 automatic)

Closing times in 3 steps

Loop detector functionality (not with MGC without detector module)

Reaction to power failure: locked, unlocked, automatic opening

Reverse after impact and subsequent reaction

Start-up behaviour (8 options)

Closing delay

Hold-open time up to 60 s

Various cut off angles

opening vend count

System extension master / slave operation

Traffic light control

Reaction to unauthorised vehicles following (tailgating)

Scope of delivery:

Prohibition signs for pedestrian passage and prohibition for motorised and non-motorised bicycles, documentation, U-profiles for attachment.

fulfilled standards:

EN ISO 12100, EN 60204, EN 61000-6-2, EN 61000-6-3, EN ISO 13849, EN 13241, EN 12453, 
testing in accordance with the Construction Products Regulation.

Technical data:

| Power supply: | 85 – 264 AC, 50/60 Hz |
| --- | --- |
| Power consumption max: | 450 W |
| Duty cycle: | 100 % (operating mode: S1) |
| Specified number of cycles: | 10 Mio. million openings / closings |
| Opening times: | 0,3 s |
| Closing times: | 0,3 s / 0,6 s / 0,9 s |
| Temperature range: | –30° C to +55° C |
| Dimensions (W x D x H): | 315 mm x 360 mm x 1,115 mm |
| IP rating: | IP 54 |
| Weight: | 44 kg |

Assembly:

On on-site concrete foundation using U-profiles (included in delivery) and 4 adhesive anchors (can be ordered separately)

Installation by a technician certified by the manufacturer

Instruction and handover of documentation including test book, Declaration of Conformity according to Machinery Directive / Low Voltage Directive / EMC Directive and Declaration of Performance according to Construction Products Regulation

Note for planners: Each passage line requires 2 pieces of Toll HighSpeed 2, which are set up with the boom tips opposite-facing and controlled in parallel. This is the only way to secure a clearance width of approx. 3.10 m to 3.20 m.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | Toll HighSpeed 2 with control unit MGC without loop detector module |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

| Type: | Toll HighSpeed 2 with control unit MGC with 2-channel loop detector module |  |  |  |
| --- | --- | --- | --- | --- |
| Quantity: | ……………… | Piece(s) | UP: …………… | TP: …………… |

Note for planners: There are 3 different alternatives available for the barrier boom and the boom flange.

Barrier boom alternative 1:

[Item: …………..]

Barrier boom MicroBoom-T with blocking width ........... m (max. 1.5 m)

Barrier boom made of round aluminium profile, diameter 75 mm, red reflective stripes on both sides every 500 mm.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MICROBOOM-Txxx |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Swing-away flange for MicroBoom-T round boom

Barrier boom is released by a vehicle in the event of horizontal impact and swings out in the direction of travel to protect the barrier boom and reduce damage to the vehicle. Barrier boom can then be manually latched back into the boom flange.

Note: For this function option boom contact BAK01 is mandatory.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | FLSAMBT |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

Barrier boom alternative 2:

[Item: …………..]

Barrier boom MicroBoom-AT015 with blocking width 1.5 m

Soft barrier boom made of round aluminium core, diameter 38 mm, coated with foam material, outer diameter approx. 80 mm, wrapped with red reflective stripes every 500 mm.

Fixed boom length for blocking width 1.5 m, cannot be shortened.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MICROBOOM-AT015 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Swing-away flange for soft MicroBoom-AT015 round boom

Barrier boom is released by a vehicle in the event of horizontal impact and swings out in the direction of travel to protect the barrier boom and reduce damage to the vehicle. Barrier boom can then be manually latched back into the boom flange.

Note: For this function option boom contact BAK01 is mandatory.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | FLSAMBXT |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

Barrier boom alternative 3:

[Item: …………..]

Barrier boom MicroBoom-AT015 with blocking width 1.5 m

Soft barrier boom made of round aluminium core, diameter 38 mm, coated with foam material, outer diameter approx. 80 mm, wrapped with red reflective stripes every 500 mm.

Fixed boom length for blocking width 1.5 m, cannot be shortened.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | MICROBOOM-AT015 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Automatic swing-away flange for soft MicroBoom-AT015 round boom

Barrier boom is released by a vehicle in the event of horizontal impact and swings out in the direction of travel to protect the barrier boom and reduce damage to the vehicle. After deflexion of the barrier boom, the barrier opens slowly and swivels the barrier boom out of the dangerous zone. The barrier boom is automatically retrieved by means of a spring. Depending on the parameter setting, the barrier subsequently closes automatically or according to a separate closing signal.

Note: For this function option boom contact BAK01 is mandatory.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | FLSAUTOMBXT |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

In addition, the following accessories are required for all barrier boom alternatives:

[Item: …………..]

Boom contact BAK01

Gives feedback to the control unit MGC / MGC Pro when the barrier boom has been released. The barrier then turns the boom flange to the open position so that the barrier boom is swivelled out of the road area.

If the control unit MGC Pro is used, the control unit can also provide an alarm at an output.

Options for toll barrier Magnetic MHTM™ MicroDrive, type Toll HighSpeed 2

[Item: …………..]

Option special colour SL01

Barrier housing and hood in the same freely selectable colour: RAL ………….. (please fill in).

Doors in standard colour (Black grey, similar to RAL 7021).

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | SL01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option special colour SL02

Barrier housing, hood and doors in the same freely selectable colour: RAL ………….. (please fill in).

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | SL02 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option special colour SL03

Barrier housing and hood in a freely selectable colour: RAL ………….. (please fill in).

Doors in other freely selectable colour: RAL ………….. (please fill in).

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | SL03 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option special colour SL04

Barrier housing in a freely selectable colour: RAL ………….. (please fill in).

Hood in freely selectable colour: RAL ………….. (please fill in).

Doors in freely selectable colour: RAL ………….. (please fill in).

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | SL04 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option attachment set BSS01

Installation material for screwing a barrier to the concrete foundation, consisting of 4 adhesive cartridges and screwing parts.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | BSS01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option second loop detector module 2-channel DM02

Pluggable module for expansion of the MGC (Pro) control to 4 loop channels,
multiplex of all 4 loops without bilateral cross talk.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | DM02 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option Ethernet module EM01

Pluggable Ethernet module for communication via MODBUS TCP/IP protocol.

Control and parameterisation of MHTM™ MicroDrive barriers, as well as export of parameters, information and operating states possible.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | EM01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option RS485 module RSM01

Pluggable RS485 module for communication via MODBUS protocol.

Control and parameterisation of MHTM™ MicroDrive barriers, as well as export of parameters, information and operating states possible.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | RSM01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option CAN module ECN01

Pluggable CAN module for communication via CANOpen protocol.

Used for counting vehicles in master-slave operation of several barriers.

Control and parameterisation of MHTM™ MicroDrive barriers, as well as export of parameters, information and operating states possible.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | ECN01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option key switch SS01

For permanent open / button function CLOSE.

Installed in the barrier housing. Design for on-site profile cylinder.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | SS01 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Optional safety light barrier for pedestrian protection LS21

With test input for connection to control unit MGC (Pro).

Tested before each closing movement to verify proper operation.

Consisting of transmitter and receiver incl. attachments.

Installed on barrier housing and support posts (must be ordered separately).

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | LS21 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option induction loop IS06
as safety loop for cars

Circumference 6 m incl. waterproof connection box and 15 m supply line, laying loops and connection to previously described barrier controller, incl. measurements and documentation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | IS06 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |

[Item: …………..]

Option induction loop IS06
as opening loop for cars

Circumference 6 m incl. waterproof connection box and 15 m supply line, laying loops and connection to previously described barrier controller, incl. measurements and documentation.

| Manufacturer: | MAGNETIC AUTOCONTROL GMBH |  |  |  |
| --- | --- | --- | --- | --- |
| Type: | IS06 |  |  |  |
| Quantity: | ……………. | Piece(s) | UP: …………… | TP: …………… |