[Item: …………..]

Pre-security Momentum® mPass

Electric motor driven wing gate (speed gate) for pre-security check at airports.

mPass is part of the Momentum® series, a modular concept for pre-security gates, self-boarding gates and automatic border control gates. It optimizes access and passenger control at all stations along the customer

journey. This considerably increases the momentum of passenger flows at airports.

Function:

- Comfortable contact free usage

- Complete opening of swing door for a convenient passage

- Quick opening times (depending on wing size)

- Low-energy mode according to EN 17352

- Automatic closing after passage

- Light ring on the drive unit for indication of gate status

- Light barriers in the foot area and in the gate hand rail monitor the passage of persons and objects (10/10 light barriers)

- Error history (logs) for technical service

- For indoor installation only (IP32)

- Alarm reset button for stopping the acoustic signal in case of a fault

Options:

- LED lights to guide the passenger through the gate integrated in the sensor bar (position near the handrail of the gate)

- LED light for floor illumination

- 10,1’’ Touchscreen with frame illumination for call to action for the passing person. The illumination indicates the status of the gate, visible from entry and exit site

- Integrated barrier belt, fixation with magnets for the blocking of the passage way

- Preparation for Biometric camera integration for passenger inspection

- Gantry to integrate additional electronic devices (e.g. sensors, displays, overhead cameras) and/or cable routing through the bar

- De-boarding button allows the passage in the opposite direction (from the exit site)

- Integration of the most common or requested readers as control elements possible 
(450 x 95 x 145 mm (L x D x W))

- Buzzer for acoustic alarm signal

- Key switch to select different operating modes (standard/biometric/manual etc.)

Design:

- Design housing with curved lines, slim silhouette and tender forms

- No visible connection elements

- Standard colours: RAL 9005 (frame) and RAL 9010 (body)

- Optional RAL colours available for the body part to adapt the gate to any airport environment

- Glass colours clear/transparent or grey tinted

- Integrated lightning elements reinforce the contour of the product

MSDS® (Magnetic Safety Drive System) - safety & security:

- Certified (TÜV Süd) low energy drive system (for maximum personal safety) with max. 1.69 J, static opening and closing force max. 67 N (according to EN 17352, Performance Level d)

- Integrated MHTMTM drive system (brushless DC motor) for at least 10 million cycles (MCBF), certified according to EN 60335-2-103

- Duty cycle S1 (continuous operation), 100% according to DIN EN 60034-1

- Automatic opening in case of emergency alarm/power failure (without springs or batteries)

- Free swinging doors (between <15 N (standard wing and <55 N (high wing)) in case of power failure and automatic approach to home position when power is restored

- Interlock blocks the swing doors in case of non-validated passage, vandalism, etc.

- No under-crawling or over-climbing through different wing height options; standard height 1100 mm, optional up to 1600 mm

- Detection of simultaneous passage of 2 persons (tailgating)

- Detection of passage in the opposite release direction (wrong way)

- 	  Detection of entry without validation (intrusion)

Electric/Software:

- Integrated MGC Plus control system with display for parameterization and set-up

- Safety paths certified according to DIN EN 13849, Performance Level d

- Function controlled via digital inputs and outputs: emergency release, direction of passage, hold-open time, direction of passage indication, lighting options, random generator, behaviour in case of emergency, warning message in case of forced passage

- Connection for 100 – 240 VAC, 50/60 Hz

- 8 digital inputs with free allocable functions

- 4 digital outputs and 6 output relays for illumination control and status readout (free allocable)

- 3 plug-in card slots for expansion modules

- USB and LAN connection for firmware update and remote access as standard

- Integrated web server for configuring the control

- Service switch, for deactivating the inputs during service work on the barrier

Material:

- Housing: powder coated or stainless steel, aluminium and glass

- Highly resistant to impacts, scratches and abrasion

- Resistant against corrosion, UV light, detergents and disinfectants.

- Enclosure protection rating: IP32 (according EN 60529)

- Blocking element: single-layer safety glass according EN 12150-1, laminated glass according EN ISO 12543-1 and -2

- Optional integrated barrier belt made of fabric with magnets for fixation

- Platform (Option): birch wood Multiplex B1 varnished with RAL 9007 gloss

Dimensions:

- Housing dimensions: 1745 x 1305 x 160 mm (L x H x W)

- Passage width: 600 mm (Standard) / 900 mm (wide lane) / Specials on request

- Wing height: 1100 mm (Standard) / Specials on request

- Total width for standard passage width: 1040 mm

- Installation space for reader devices: 450 x 95 x 145 mm (L x D x W)

Installation:

- Screw mounting on stable floor surface

- Alternative glue mounting by use of a metal base plate (accessory)

- Platform (accessory)

Temperature range: 		0 to +40 °C

Weight:				Unit A 110kg; Unit B 140Kg; Unit C: 160Kg

Installation execution by manufacturer trained installation specialist, including user training and hand-over of inspection manual, operating manual, test book and declaration of conformity according to EC machinery directive/ low voltage directive, EMC directive.

Momentum® mPass

[Item: …………..]

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	mPass A unit, MPA_AU0011

For passage width of: …………..				Wing height: …………..

Material of wing element (glass/ acrylic): …………..

Number: …………..      Unit: pieces			Unit price: …………..	Total: …………..

[Item: …………..]

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	mPass B unit, MPA_BU0011

For passage width of: …………..		For passage width of: …………..

Material of wing element (glass/ acrylic): …………..

Number: …………..      Unit: pieces			Unit price: …………..	Total: …………..

[Item: …………..]

Manufacturer:	Magnetic Autocontrol GmbH

Type: 	mPass C unit, MPA_CU0011

For passage width of: …………..				Wing height: …………..

Wing height: …………..		Material (glass/ acrylic): …………..

Number: …………..      Unit: pieces			Unit price: …………..	Total: …………..

Option

[Item: …………..]

Manufacturer:	Magnetic Autocontrol GmbH

Type:

Number: …………..      Unit: pieces		Unit price: …………..	Total: …………..