Magnetic turnstiles with options and accessories

[Position: …………..]

Mechanical Magnetic turnstile, MPT330CZ300

Mechanical turnstile for access control and single passage pedestrian separation.

Function:

- The MPT330* turnstile is used to control pedestrians, mainly in outdoor areas with relatively high security requirements.

- Smooth-running mechanism for comfortable passage

- Spring-assisted rotation after light touch

- Direction of operation freely selectable for entry and exit and manually selectable (via lock)

- The entire turnstile can also be locked manually (via lock)

- Lock included in delivery

- Passage cycles (MCBF) min. 10 million

- Safety and minimum distances in accordance with EN17352

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Electromechanical turnstile Magnetic, MPT333CA300

Electromechanical turnstile for access control and single passage pedestrian separation.

Function:

- The MPT333* turnstile is used to control pedestrians, mainly in outdoor areas with relatively high security requirements.

- Smooth-running electromechanics for comfortable passage

- Spring-assisted rotary movement after release pulse and light touch of the hand

- 2-way operation in entry and exit directions

- The turnstile is supplied with a "voltage-free free-turning lock" as standard.

- The "locked when de-energised" configuration can be specified when ordering or easily converted in the field.

- Signal deletion when not in use after a set time

- Pulse counting in both directions of passage

- Error history (logs) for technical service

- Integrated random generator for person and bag checks

- The "right-hand side of entrance" version is supplied as standard; "left-hand side of entrance" must be specified on the order

- In the event of forced passage, the barrier sends a warning message

- In the event of a power failure or alarm signal, the barrier can be rotated freely in both directions

- Duty cycle S1 (continuous operation), 100% in accordance with DIN EN60034-1

- Passage cycles (MCBF) min. 10 million

- Safety and minimum distances in accordance with EN17352

Electrics/software:

- Integrated MGC control with display for parameterisation and commissioning

- Functions and parameters can be activated and adjusted and switched via digital inputs and outputs: Direction of passage, hold-open time, direction of passage indicator, random generator

- Connection for 100–240 VAC, 50/60 Hz

- Control with activation by reception (PULT02) and with all common card readers

- 8 digital inputs with freely assignable functions

- 4 digital outputs and 6 output relays for status reporting (freely assignable)

- 5 plug-in card slots for expansion modules

- Can be networked via Ethernet interface

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Magnetic motorised turnstile, MPT332CA300

Motorised turnstile for access control and single passage pedestrian separation.

Function:

- The MPT332* turnstile is used to control pedestrians, mainly in outdoor areas with relatively high security requirements.

- After an authorisation pulse (e.g. access control device), the turnstile indicates readiness by briefly turning the centre column (approx. 2°).

- A brushless electric motor supports the further rotational movement after a light touch and guides the user comfortably onwards.

- 2-way operation in entry and exit directions

- The turnstile is supplied with a "voltage-free free-turning lock" as standard.

- The "locked when de-energised" configuration can be specified when ordering or easily converted on site.

- Signal deletion when not in use after a set time

- Pulse counting in both directions of travel

- Error history (logs) for technical service

- Integrated random generator for personnel and bag checks

- The standard version is delivered with the entrance on the right; if you require the entrance on the left, please specify this on your order

- In the event of forced passage, the barrier sends a warning message

- In the event of a power failure or alarm signal, the barrier can be rotated freely in both directions

- When de-energised, the centre column cannot be turned manually; this has been confirmed by a risk assessment

- Duty cycle S1 (continuous operation), 100% in accordance with DIN EN60034-1

- Passage cycles (MCBF) min. 10 million

- Safety and minimum distances in accordance with EN17352

Drive system/safety

- Certified (TÜV SÜD) low-energy drive (for maximum personal safety) up to max. 4J static force when opening and closing max. 67N

(in accordance with requirements for personnel barriers EN17352:2022, EN13849.1 Performance Level d cat.2)

- Integrated MHTMTM drive system (brushless DC motor with planetary gear) for at least 10 million passage cycles (MCBF), certified in accordance with EN 60335-2-103

Electrics/software:

- Integrated MGC control with display for parameterisation and commissioning

- Functions and parameters can be activated and adjusted and switched via digital inputs and outputs: direction of passage, hold-open time, direction of passage indicator, random generator

- Connection for 100–240 VAC, 50/60 Hz

- Control with activation via reception (PULT02) and with all common card readers

- 8 digital inputs with freely assignable functions

- 4 digital outputs and 6 output relays for status reporting (freely assignable)

- 5 plug-in card slots for expansion modules

- Can be networked via Ethernet interface

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

Cross-type product information for the following versions

Mechanical MPT330CZ300 / Electromechanical MPT333CA300 / Electromotive MPT332CA300

Design / Material:

- Self-supporting steel construction (support beams, service door, centre column, cage half, locking comb), hot-dip galvanised and then powder-coated in RAL7042 traffic grey

- Other RAL colours are available as an option

- High impact, scratch and abrasion resistance

- Centre column in V4A stainless steel quality (AISI316L; AISI316Ti) available as an option

- Cage half, locking comb in stainless steel quality V4A (AISI316L; AISI316Ti) available as an option

- Bottom bearing of the centre column with a low-maintenance PTFE plain bearing, 		        no relubrication required

Dimensions:

- MPT*3* with 3x 120° centre column 	1510 x 1288 x 2235 mm, (L x W x H)

- MPT*3* with 4x 90° centre column 		1359 x 1076 x 2235 mm, (L x W x H)

Temperature range: 		-30 to +55 °C

Weight:				approx. 250 kg (without options and accessories)

Protection class: 			IP 43 (IP54 available as an option)

Duty cycle (ED):			100

Designed for 10,000,000 passenger passages

Throughput

(Depending on reader, product configuration and user behaviour): approx. 20 people per minute1

Installation

- Installation on load-bearing floor/foundation using composite anchors included in delivery

- Alternative installation using foundation frame (available as an accessory), required in conjunction with composite or paving stones

Installation by a technician trained by the manufacturer

instruction and handover of documentation including test book, operating instructions, declaration of conformity in accordance with the Machinery Directive / Low Voltage Directive / EMC Directive

Options for MPT *3* turnstile with single passage

[Position: …………..]

Option MPT*3* turnstile pre-assembled at the factory; HELAFUT3PRE

Turnstile on FURA foundation frame, including 4 lifting lugs for easy transport by forklift

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option additional lifting lugs; HELAFUT3

Consisting of 2 pieces, for extension for customer platform or running surface

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: pcs		EP: …………..	GP: …………..

[Position: …………..]

Special colour option

Complete steel construction of the MPT*3* turnstile, powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option: centre column in stainless steel

Centre column in stainless steel V4A (AISI316L; AISI316Ti), steel construction of the turnstile MPT*3* powder-coated in RAL7042

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option Centre column in stainless steel, steel construction in special colour

Centre column in stainless steel V4A (AISI316L; AISI316Ti), steel construction of the turnstile MPT*3* powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option Turnstile in stainless steel

Centre column, cage half, locking comb of the MPT*3* turnstile in stainless steel V4A (AISI316L; AISI316Ti),

support beam and service door as steel construction powder-coated RAL7042

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option centre column in 4x 90°

Turnstile MPT*3* with centre column in 4x 90° division as steel construction powder-coated RAL7042

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Option profile half cylinder; PHZT3GBMP

Preparation of profile half cylinder on service door of support beam (MPT*3*/MPG/MPB)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Locking option (MPT333*); EP100

Conversion fee Input locked in the event of a power failure

Required once per direction of travel

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Locking option (MPT333*); EP101

Conversion fee Output locked in the event of a power failure

Required once per direction of travel

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Locking option (MPT333*); MAENT101

MPT mechanical unlocking lock (per direction)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Lock segment option (cage half with locking bar); KH01

Barrier segment (cage half with barrier bar) for MPT*3*, hot-dip galvanised and powder-coated in RAL7042

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Lock segment option (cage half with locking bar); KH0X

Lock segment (cage half with locking bar) for MPT*3* hot-dip galvanised and powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

Accessories for MPT turnstile *3*

[Position: …………..]

Accessories for GED (Gate End Display) passage indicator; GED105MP

2 gate end displays (GED) for MPT*3*/MPG/MPB, green arrow, red X, one on each side, integrated into the support beam/cover

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: pcs		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessory mounting column for the installation of access controls; ASMP

Mounting column made of chrome steel, powder-coated RAL7042, incl. mounting plate

Dimensions: 704 x 260 x 260 mm, installation area max. 600 x 160 mm

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessory mounting column for the installation of access controls; ASMPX

Mounting column made of chrome steel, powder-coated in RAL special colour, incl. mounting plate

Dimensions: 704 x 260 x 260 mm, installation area max. 600 x 160 mm

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessory mounting column for the installation of access controls; ASMPPHZ

Mounting column made of chrome steel, powder-coated RAL7042, incl. mounting plate, prepared for profile half cylinder (PHZ)

Dimensions: 704 x 260 x 260 mm, installation area max. 600 x 160 mm

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessory mounting column for the installation of access controls; ASMPPHZX

Mounting column made of chrome steel, powder-coated in RAL special colour, incl. mounting plate, prepared for PHZ, dimensions: 704 x 260 x 260 mm, installation area max. 600 x 160 mm

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessories Reader holder cover; AMWMPTR

Reader holder cover Stainless steel, powder-coated in RAL7042, 1 piece

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: pcs		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessories Reader holder cover; AMWMPTX

Reader holder cover Stainless steel, powder-coated in special colour Turnstile, 1 piece

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: pcs…..	EP: …………..	GP: …………..

[Position: …………..]

Accessories Reader holder cover; AMWMPT

Reader holder cover Stainless steel, uncoated, 1 piece

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: pcs		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessories Push button; DT101

Illuminated push button for enabling passage by pressing the button

(per passage direction)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessories Push button; DT102

Surface-mounted push button IP67 for free output

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Lighting accessories; BELED

LED lighting on both sides for MPT*3*/MPG/MPB, with 2 lights, powder-coated in RAL7042, without twilight switch (not for mechanical high turnstiles, only for 230V/50Hz)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Lighting accessories; BELEDDS

LED lighting on both sides for MPT*3*/MPG/MPB, with 2 lights, powder-coated in RAL7042, with twilight switch (not for mechanical high turnstiles, only for 230V/50Hz)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; ADACHT3R

Raised roof for MPT*3*, powder-coated in RAL 7042

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; ADACHT3FX

Raised roof for MPT*3*, powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; ADACH2T3R

Raised roof for 2 MPT*3* side by side, powder-coated in RAL7042

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; ADACH2T3FX

Raised roof for 2 MPT*3* side by side, powder-coated in special colour

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACHT3R

Free-standing pent roof for MPT*3*, powder-coated in RAL7042 (4 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACHT3FX

Free-standing pent roof for MPT*3*, powder-coated in special colour (4 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACH2T3R

Free-standing pent roof for 2 MPT*3*, powder-coated in RAL7042 (6 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACH2T3FX

Free-standing pent roof for 2 MPT*3*, powder-coated in special colour (6 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACH3T3R

Free-standing pent roof for 3 MPT*3*, powder-coated in RAL7042 (8 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; PDACH3T3FX

Free-standing pent roof for 3 MPT*3*, powder-coated in special colour (8 posts)

Manufacturer:	Magnetic Autocontrol GmbH

Colour type: …………..    Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; BSS104

Mounting kit for pent roof on foundation frame, set for 2 posts

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium roof accessories; BSS105

Fastenings kit for pent roof on ground, set for 2 posts

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessories Premium foundation frame; FURAT3

Foundation frame for MPT*3* without roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Premium foundation frame accessories; FURAT3D

Foundation frame for MPT*3* with pent roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		Packaging unit: …………..	GP: …………..

[Position: …………..]

Accessories Premium foundation frame; FURAAD2T3

Foundation frame for 2x MPT*3* with roof structure

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Accessories Premium foundation frame; FURA2T3D

Foundation frame for 2x MPT*3* with pent roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium foundation frame accessories; FURA3T3D

Foundation frame for 3x MPT*3* with pent roof

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..

[Position: …………..]

Premium foundation frame accessories; KHADS100

Adapter set for FURAT3/FURAT3D for cage half with locking bar

Manufacturer:	Magnetic Autocontrol GmbH

Quantity: …………..      Unit: St		EP: …………..	GP: …………..